# Configurable AMBA 5 CHI Interconnect

Design Specification

CHI RTL project

Baseline: Arm IHI 0050H, Issue H

Contents

List of Tables

List of Figures

# DOCUMENT STATUS

This document describes the RTL in this repository against Arm IHI 0050H. It is a reference implementation, not a compliance claim. Every statement about behaviour below was checked against the source, and the feature matrix in Section 1.3 states exactly where each function stops.

The document is generated from a single Markdown source in doc-src/ so that the Word file in docs/ and the RTL never drift apart. See Section 12.7 for the regeneration command.

# Chapter 1
	SCOPE, ISSUE H BASELINE, AND FEATURE STATUS

## 1.1 Baseline and exclusions

The architectural reference is Arm IHI 0050H, AMBA CHI Architecture Specification, Issue H, 17 September 2025: https://developer.arm.com/documentation/ihi0050/h/. Section references identify relevant Issue H material without reproducing Arm prose or tables.

The project implements an on-chip CHI interconnect among RN-F, RN-D, RN-I, SN-F, SN-I, HN-F, HN-I and MN functions. The following are outside the project scope and are not present in any form:

- CHI Chip-to-Chip (IHI 0098), CXS, CCIX, die-to-die framing, C2C containers and link services.

- Device Assignment and Coherent Device Assignment protocol behaviour. The RME-CDA StreamID and SecSID1 flit fields are carried, but no assignment or ownership checking exists.

- GDI.

- An RN-side cache controller. This repository is the interconnect; the RN-F line-state model in Section 10 is the contract an RN must meet, not RTL that is shipped here.

The Issue H errata have not been assessed against this RTL. An item marked Implemented below means the named block exists in synthesizable RTL and performs the described function; it is not an interoperability statement.

## 1.2 Status definitions

Implemented — the function exists in synthesizable RTL and performs the described protocol behaviour over its stated contract.

Partial — useful logic exists, but protocol cases, integration, or end-to-end behaviour are incomplete. The boundary column says which.

Not implemented — no semantic implementation exists. An enum value or a packed flit field by itself is carriage, not implementation.

## 1.3 Feature matrix

| Feature | Issue H reference | Status | Actual RTL boundary |
| --- | --- | --- | --- |
| REQ/RSP/SNP/DAT records and pack/unpack | B13.9, B13.10 | Implemented | Configurable packed records match the Issue H field lists and slot widths, including every documented field overlay. Opcode-dependent field legality is not policed. |
| Fixed flit slot widths and overlays | B13.9.1–B13.9.4 | Implemented | TxnID 12, ReturnTxnID/StashLPID 12, NumReq/Size 6, GroupID 8, RSP DBID 12, DAT DBID 16, DataSource 8, CCID 2, DataID 2, CacheLineID 6, PAS 3 are slot widths independent of the node-side field widths. |
| PAS[2:0] replacing NS/NSE | B2.9, B13.10 | Implemented | REQ and SNP carry the three-bit Physical Address Space selector and the SAM decodes its non-secure sense. |
| One node-facing interface type | B13 | Implemented | if/chi_if.sv is the only interface declaration in the repository and is a superset used by all five external node-class arrays. |
| Top-level interface count and replicated channels | B13.7 | Not implemented | Elaboration requires numIfPerNode == 1 and replChannels == 1. No top-level replication exists. |
| L-Credit manager, link TX and link RX | B13.11, B14.2 | Implemented | chi_lcrd_mgr, chi_link_tx and chi_link_rx implement finite credit accounting, receive buffering and credit return. |
| Link layer integrated at every node port | B13.11, B14.2 | Implemented | chi_link_port instantiates three receivers and three or four transmitters per port, and chi_interconnect_top instantiates one per RN-F, RN-D, RN-I, SN-F and SN-I interface. *LCRDV are real credit pulses, not ready echoes. |
| Link activation and deactivation FSM | B14.5–B14.6 | Implemented | chi_link_fsm implements STOP, ACTIVATE, RUN and DEACTIVATE with credit-drain conditions, and is instantiated per channel per direction inside every port. |
| System coherency interface | B14.4 | Partial | TXSACTIVE and RXSACTIVE follow reset and SYSCOACK echoes SYSCOREQ. There is no power-domain or coherency-entry sequencing. |
| Flit parity | B9.3 | Implemented | chi_link_tx generates per-byte flit parity and chi_link_rx checks it. parityEn is accepted at the top and a mismatch raises linkParityError for that node. |
| Control and credit signal checks | B9.3 | Not implemented | The *CHK scalars beside control, valid and credit signals are declared in the interface and driven to zero. Only the flit parity vectors are live. |
| Link error reporting | B9 | Implemented | linkUp, linkParityError and linkCreditError are per-node-ordinal top-level outputs. Credit error covers receive-buffer overflow and L-Credit overflow or underflow. |
| Four physically separated internal channels | B2.1, B13 | Implemented | REQ, RSP, SNP and DAT have separate ready/valid arrays, arbitration and topology storage throughout. |
| Crossbar | B1.2 | Implemented | Per-output, per-channel QoS, age and round-robin arbitration. |
| Bidirectional ring | B1.2 | Implemented | Shortest-direction routing, clockwise tie-break, directional registers, local and transit alternation, and ejection round-robin. |
| 2D mesh with deterministic XY routing | B1.2, B3 | Implemented | Per-channel routers, one output register per crosspoint output, and placement-driven device ports. |
| Per-hop L-Credits inside the fabric | B13.11 | Not implemented | Internal topology links use ready/valid with registered crosspoint outputs. CHI L-Credits terminate at the node ports. |
| Requester SAM, HN striping and HN-I SAM | B3.1–B3.3 | Implemented | Ordered region table, security rejection, XOR-fold hash and runtime update ports. |
| PrefetchTgt routing | B3.3.1, B4.2.6.2 | Implemented | chi_prefetch_map maps the address to an SN-F independently of the HN hash, the request bypasses the Home, and both the HN-F and HN-I accept and discard it without a response and without a RetryAck. |
| Coherent read family | B4, B5.1 | Partial | ReadShared, ReadClean, ReadNotSharedDirty, ReadUnique, ReadOnce, ReadPreferUnique, ReadOnceCleanInvalid and ReadOnceMakeInvalid are classified, snooped when required, forwarded, and completed. Response-state selection is simplified and not every legal state combination is produced. |
| Write family | B5.3 | Partial | Fourteen writes including the stash forms are classified and relayed with CompDBIDResp combining. Partial-write byte-enable rules, write cancellation and every combined form are incomplete. |
| CMO and combined write-CMO | B7 | Partial | Opcode classification plus completion and persist tracking. Architectural completion points and the snoop and data side effects of each CMO are incomplete. |
| DCT | B2.5.1.3, B4.8 | Partial | Eligibility, forwarding-snoop generation and SnpRespFwded collection exist. The fallback path when the forwarding peer cannot supply data is not implemented. |
| DMT, including ReadNoSnpSep | B2.5.1.1–B2.5.1.2 | Implemented | chi_dmt_ctrl selects direct memory transfer, the downstream read names the Requester in ReturnNID and ReturnTxnID, the Subordinate sends CompData straight to the Requester, and the Home issues only the response it still owes. |
| DWT | B2.3.2.1, B13.10.24 | Implemented | chi_dwt_ctrl sets DoDWT on the downstream write, the Subordinate returns DBIDResp directly to the Requester, and the Home forwards only the completion. |
| Atomics | B2.3.3, B4.2.5, B5.4 | Implemented | chi_atomic_alu implements the AtomicStore, AtomicLoad, AtomicSwap and AtomicCompare operations. The Home serializes the operand, reads the initial value, applies the operation, writes it back and returns the original value. Little-endian and big-endian operands are both handled. The datapath is 64 bits, so operands wider than 8 bytes are not supported. |
| Far atomics | B4.2.5 | Partial | Operations are executed at the Home. There is no negotiation that pushes the operation to the Subordinate. |
| DVM and MN | B2.3.7, B8 | Partial | The MN serially unicasts SnpDVMOp to configured Requesters and collects responses. The two-part DVM message payload is not decoded, so operation-specific ordering is absent. |
| Sequential coherent snoops | B4.3–B4.4, B4.8 | Partial | The Home walks a precise sharer vector one RN-F at a time and collects responses including forwarded and dirty-data cases. One snoop is outstanding at a time. |
| Cache stashing | B4.9, B7 | Implemented | chi_stash_ctrl decodes WriteUniqueFullStash, WriteUniquePtlStash, StashOnceShared, StashOnceUnique, StashOnceSepShared and StashOnceSepUnique, resolves StashNID, StashNIDValid, StashLPID and StashLPIDValid, issues the matching stash snoop, honours a DataPull in the snoop response by issuing the implied read, and returns Comp, StashDone or the combined CompStashDone. |
| RN cache states and transitions | B4.1, B4.6–B4.8 | Not implemented | Out of project scope. State encodings exist in chi_pkg and the required model is documented in Section 10, but there is no RN cache controller here. |
| POS and same-line hazards | B1.5, B2.7, B4.11 | Partial | POS storage, allocation and same-line plus ordered hazard logic exist, but the Home FSM admits one transaction at a time, so configured POS depth does not create concurrency. |
| Inclusive precise snoop filter | B4.4, B4.11 | Partial | Set-associative sharer and owner storage with rotating replacement and sequential back-invalidation. State updates are simplified and are not a complete Issue H directory model. |
| Request Retry, RetryAck and PCrdGrant | B2.5.5, B2.10 | Implemented | chi_retry records the SrcID, QoS and credit class behind every RetryAck, allocates one of four P-Credit types, and issues PCrdGrant naming the recorded Requester with no TxnID and no DBID. QoS selects which record is granted and age breaks ties. |
| P-Credit return | B2.5.6, B2.10.1 | Implemented | A granted credit leaves the accounting either when a request arrives with AllowRetry deasserted or when the Requester returns it with PCrdReturn, which the Home accepts and discards. |
| Exclusives | B6 | Partial | Line and requester reservation monitors produce EXOK. Request classification and the full set of invalidation and failure rules are simplified. |
| Ordering and ReadReceipt | B2.7 | Partial | Ordered requests are globally serialized and a ReadReceipt is returned for a request that asks for one. Endpoint order and request order are not distinguished. |
| DataCheck and poison carriage | B9, B13.10 | Partial | Optional DAT bits pack and unpack end to end. There is no error-response generation from a data check failure. |
| MultiReq, NumReq and CacheLineID | B2.8, B13.10 | Not implemented | The fields and the Size/NumReq slot overlay are carried, but no multi-request expansion or per-line tracking exists. multiReqEn is checked metadata. |
| PBHA | B13.10 | Not implemented | The four-bit field and its REQ, SNP and DAT slot overlays are carried. No page-based hardware attribute behaviour exists. |
| MTE tag semantics | B12, B13.10 | Not implemented | Tag, TU and TagOp are carried. There is no allocation-tag storage, matching or tag-operation behaviour. |
| MEC | B10.6, B13.10 | Not implemented | The 16-bit MECID and its DAT slot overlay are carried. No MEC access control, translation or consistency checking exists. |
| MPAM | B11.4 | Partial | The 12-bit or 15-bit bus is carried in REQ and SNP. There is no resource partitioning or monitoring. |
| APB configuration | Project-defined | Implemented | Secure-write staged region editor with a quiescent commit. Limitations are in Section 7. |
| CHI MMIO configuration | Project-defined | Partial | The HN-I recognizes a 4 KiB window and simple read and write offsets. Security, validation and quiescent commit are incomplete. |
| SVA | Sections above | Partial | Bound local safety, stability and protocol-field properties with functional covers. There is no full protocol checker and no proof closure. |

Table 1.1 Feature status against Arm IHI 0050H

# Chapter 2
	ARCHITECTURE

## 2.1 Architecture at a glance

Fig 2.1 Top-level architecture. The vertical axis is the request path.

Read the figure top to bottom as the life of a request. A Requester drives a REQ flit onto its chi_if. The chi_link_port for that port converts the CHI link-layer contract, which is credit-based and has no ready signal, into the ready/valid handshake the fabric uses internally: it will not accept a flit unless the link is in RUN and an L-Credit is outstanding, and it checks flit parity on the way in. The address then goes to that port’s chi_sam, which finds the matching region, applies the PAS check and produces a TgtID — through chi_hn_hash for an interleaved coherent region, or through chi_prefetch_map for a PrefetchTgt hint, which is routed straight to an SN-F because no Home resource may be consumed for it. chi_noc_top carries the flit across whichever of the three topologies was elaborated, on one of four independent networks. A Home Node serializes it, and anything it must send downstream leaves through a second chi_link_port to a Subordinate.

Three properties of this arrangement are worth stating explicitly, because they are the ones that constrain everything else.

The link layer is per port and symmetric. Every external port gets the same four blocks, so credit accounting and activation are a property of the port rather than of the node class. Only the channel set differs: a Requester port receives REQ, RSP and DAT and transmits RSP, SNP and DAT, while a Subordinate port receives RSP and DAT and transmits REQ, RSP and DAT.

HN-F, HN-I and MN are internal fabric endpoints, not external chi_if ports. They have no link layer, no L-Credits and no activation handshake, which is why their ordinals read zero in the linkUp output. A request reaches them over the internal ready/valid fabric only.

The SAM is per Requester port, not central. Each port decodes its own address, which is what B3.1 describes when it says Requesters must have a System Address Map. A single chi_cfg_slave broadcasts region updates to all of them so they stay consistent.

The rest of this chapter takes those pieces in the order a request meets them: the node population and how it is numbered (Section 2.2), the four channels a node port presents (Section 2.3), the four layers the design is built from (Section 2.4), the interior of the coherent Home because that is where nearly all protocol state lives (Section 2.5), the configuration and status plane that sits beside all of it (Section 2.6), and then the RTL hierarchy and the handshake rules the whole design obeys (Sections 2.7 and 2.8).

## 2.2 Node classes and the node ordinal space

Eight node classes exist. Five of them are external: they appear as a chi_if port array on chi_interconnect_top and are driven by logic outside this repository. Three are internal: they are protocol endpoints built inside the fabric, reached over the internal networks, and have no pins.

| Node class | Count field | External port | Ordinals | Role in this design |
| --- | --- | --- | --- | --- |
| RN-F | numRnf | rnf[] | 0–3 | Fully coherent Requester. Issues the coherent read, write and CMO families, receives snoops, and is the only class the snoop filter tracks. |
| RN-D | numRnd | rnd[] | none | Requester that is DVM capable but has no coherent cache. Receives SnpDVMOp from the MN, and no other snoop. |
| RN-I | numRni | rni[] | 4 | I/O coherent Requester. Issues the ReadOnce and WriteUnique families, and is never snooped. |
| SN-F | numSnf | snf[] | 5–6 | Subordinate for coherent memory. Terminates the downstream reads and writes a Home issues, and is the target of PrefetchTgt. |
| SN-I | numSni | sni[] | 7 | Subordinate for peripheral space. Same channel set as SN-F; the distinction is which SAM region kind selects it. |
| HN-F | numHnf | internal | 8–11 | Coherent Home. Serialization point, snoop filter, POS, retry credits and the offload engines. Section 2.5 and Section 9. |
| HN-I | numHni | internal | 12 | Non-coherent Home. Relays to SN-I through its own SAM and hosts the CHI MMIO configuration window. |
| MN | numMn | internal | 13 | Misc Node. Sequences DVM operations to the DVM-capable Requesters. |

Table 2.1 Node classes, their count fields, and their ordinals in the default configuration

All eight classes share one flat ordinal space. The ordinal of a node is its index into the concatenation RN-F, RN-D, RN-I, SN-F, SN-I, HN-F, HN-I, MN, in that order, which is what OFF_RNF through OFF_MN compute in chi_interconnect_top. That single numbering is used for three different things, and they are worth separating:

- It indexes the internal networks. Every channel array inside the fabric is [NUM_NODES-1:0], so an ordinal is also a transport port number.

- It indexes NODE_IDS, the parameter that gives each node its CHI NodeID. Nothing in the protocol requires the two numbering schemes to agree, and B3.2 makes NodeID assignment implementation defined, so the ordinal is a fabric-internal name and the NodeID is the protocol-visible one.

- It indexes linkUp, linkParityError and linkCreditError. Since the three internal classes have no link, the top six bits of those vectors in the default configuration are constantly zero, and the smoke run in Section 12.7 asserts exactly that.

The default configuration is 4 RN-F, 1 RN-I, 2 SN-F, 1 SN-I, 4 HN-F, 1 HN-I and 1 MN: fourteen nodes, of which eight have external links.

## 2.3 The four channels

CHI defines four channels, and this design keeps them physically separate from the node pins all the way to the protocol endpoints: separate valid, separate ready, separate flit storage, separate arbitration and separate credit accounting. Nothing is time-multiplexed onto a shared bus anywhere in the fabric.

| Channel | At a Requester port | At a Subordinate port | What it carries | Default flit width |
| --- | --- | --- | --- | --- |
| REQ | node to interconnect | interconnect to node | Requests: opcode, address, Size, TxnID, SrcID, TgtID, PAS, QoS, and the stash, DVM and atomic qualifiers. | 153 |
| RSP | both directions | both directions | Responses without data: completions, DBIDResp, RetryAck, PCrdGrant, ReadReceipt, snoop responses. | 79 |
| SNP | interconnect to node only | not present | Snoops, including the forwarding and stash forms and SnpDVMOp. A Subordinate is never snooped. | 106 |
| DAT | both directions | both directions | Data, with BE, DataID, CCID, and the optional DataCheck, Poison and RSVDC fields. | 437 |

Table 2.2 The four channels, their direction at each port type and their default flit widths

The widths are the derived values for the default configuration and come from the width functions in chi_pkg; Section 3.2.1 gives the formula and the cost of each parameter that feeds it.

The separation is not cosmetic. A response must be able to make progress while a request is blocked, or the protocol deadlocks: a Home that cannot issue Comp because the REQ network ahead of it is full, on a REQ network that cannot drain because the Requester is waiting for that Comp, is a cycle with no escape. Four independent networks break that cycle structurally, which is why the transport layer in Section 2.4.3 replicates its whole datapath four times rather than sharing one wider one. The same reasoning applies one level down at the link layer, where each channel has its own L-Credit pool of lcrdDepth entries and its own activation FSM, so one starved channel cannot stall another.

SNP asymmetry is the one place where the port types differ in channel count. A Requester port has six channel instances, a Subordinate port five. Section 4.1 explains why both still use the same chi_if type.

## 2.4 The four layers

Every block in the design belongs to one of four layers. The layer boundaries are also the points at which the representation of a transaction changes, which is the useful way to read the hierarchy.

| Layer | Modules | Instances | State it owns |
| --- | --- | --- | --- |
| Link | chi_link_port, chi_link_rx, chi_link_tx, chi_lcrd_mgr, chi_link_fsm | 8 ports | L-Credit pools, receive buffers, activation state, parity |
| Address decode | chi_sam, chi_hn_hash, chi_prefetch_map | 5 Requester-side, 1 in the HN-I | Region table, hash configuration |
| Transport | chi_noc_top, and chi_xbar, chi_ring or chi_mesh | 1, four networks wide | Per-hop flit registers, arbiter age and round-robin state |
| Protocol | chi_hnf_top, chi_hni_top, chi_mn_dvm | 4, 1, 1 | POS, snoop filter, retry records, exclusive monitors, transaction FSM state |

Table 2.3 The four layers, their modules, their instance counts in the default configuration, and the state each owns

### 2.4.1 Link layer

The link layer is the only part of the design that touches CHI pins, and its job is to convert between two handshakes that do not resemble each other.

On the CHI side a transmitter may send a flit only if it holds an L-Credit that the receiver previously granted on *LCRDV, and there is no ready signal to push back with. On the fabric side everything is ready/valid. So each chi_link_port contains, per channel and per direction, one activation FSM (chi_link_fsm, with STOP, ACTIVATE, RUN and DEACTIVATE states) plus either a receiver or a transmitter:

- chi_link_rx buffers an incoming flit, checks its parity, and returns an L-Credit on *LCRDV when the buffer slot frees. The credit it returns is the node’s permission to send the next flit, so the depth of that buffer and the credit pool are the same number, lcrdDepth.

- chi_link_tx holds the credit pool for its direction in chi_lcrd_mgr, generates per-byte flit parity, and will not assert *FLITV unless the link is in RUN and a credit is available. It also detects pool overflow and underflow, which is what raises linkCreditError.

Two consequences of this structure show up elsewhere in the document. First, because the credit pool is per channel per direction, lcrdDepth multiplies by roughly the number of channels times the number of ports when you size it; Section 3.2.3 does that arithmetic. Second, because activation is per channel, a link is “up” only when every channel in both directions has reached RUN, which is the condition linkUp reports and the condition the smoke run in Section 12.7 waits for.

### 2.4.2 Address decode

A REQ flit arrives carrying an address and no target. Something has to turn that address into a TgtID before the transport layer can route it, and B3.1 puts that duty on the Requester side: every Requester must have a System Address Map. This design follows that literally, so each of the five Requester ports has its own chi_sam instance rather than sharing a central decoder.

chi_sam walks an ordered region table, takes the first matching region, and applies the region’s security rule against the incoming PAS. What happens next depends on the region kind:

- A plain region names one target directly.

- An interleaved coherent region hands the address to chi_hn_hash, which folds address bits above hnHashShift with XOR and selects one of the HN-F instances, so that consecutive interleaveGranule blocks land on different Homes.

- A PrefetchTgt request goes to chi_prefetch_map instead, which maps the address straight to an SN-F. The Home is bypassed entirely, because a prefetch hint must not be allowed to consume a Home resource that a real transaction needs.

The HN-I carries a sixth chi_sam instance, used for the second decode from the Home to an SN-I. Section 6 covers the region table and a worked hash example.

### 2.4.3 Transport

chi_noc_top is the transport layer and presents the same interface regardless of which topology was elaborated: NUM_NODES ingress ports and NUM_NODES egress ports, on each of four independent networks, all ready/valid. One of three topologies is built:

- chi_xbar gives every ingress a path to every egress, with per-output per-channel arbitration that considers QoS first, then age, then round-robin. Latency is one cycle and there is no head-of-line blocking between outputs, but the crosspoint count grows with the square of the node count.

- chi_ring places the nodes on a bidirectional ring, routes each flit the shorter way with a clockwise tie-break, and alternates between local injection and transit traffic so an injecting node cannot starve traffic passing through it.

- chi_mesh builds a meshX by meshY grid of crosspoints, each holding one chi_router per channel, with numDevPortsPerXp device ports per crosspoint. Routing is deterministic XY: all X hops first, then all Y hops, which is what makes it deadlock free without needing virtual channels for routing.

What the topologies have in common matters as much as how they differ. All three are ready/valid with registered outputs, and none of them carries CHI L-Credits internally: credits terminate at the node ports. Inside the fabric backpressure is a ready signal, which is why Section 2.8’s handshake rules are stated as a design constraint rather than a coding preference.

### 2.4.4 Protocol endpoints

The protocol layer is where a transaction stops being a flit in transit and becomes state. There are three endpoint types:

- chi_hnf_top, the coherent Home, is the serialization point for a line. It owns the snoop filter, the POS, the retry records and every engine that acts on a coherent transaction. Section 2.5 shows its interior.

- chi_hni_top, the non-coherent Home, has no coherence state at all. It decodes the address a second time through its own chi_sam, relays the transaction to an SN-I, and recognises the CHI MMIO configuration window described in Section 7.2. It also accepts and discards PrefetchTgt without responding, for the same reason the HN-F does.

- chi_mn_dvm, the Misc Node, is the DVM sequencer. It unicasts SnpDVMOp to each DVM-capable Requester in turn and collects the responses before completing the operation.

## 2.5 Home Node internal architecture

Fig 2.2 chi_hnf_top internal structure. One request is in flight at a time.

The coherent Home is where nearly all protocol state lives, so it is shown separately. A request is either admitted into chi_pos or handed to chi_retry, which is the only path that can answer with RetryAck and later PCrdGrant. An admitted request is checked by chi_hazard against every other active POS entry for a same-line or ordering conflict before the FSM acts on it. One globally serial FSM of 22 states then drives whichever engine the opcode selects: the directory and snoop generator for coherence, the DMT, DWT and stash controllers when the transaction can be offloaded so that data bypasses the Home, and the atomic, cache-maintenance and exclusive-monitor blocks for operations the Home performs itself.

The three offload engines are worth separating from the rest, because they exist to keep data out of the Home rather than to add behaviour to it. chi_dmt_ctrl names the Requester in ReturnNID and ReturnTxnID on the downstream read so the Subordinate sends CompData straight back to the Requester. chi_dwt_ctrl sets DoDWT on the downstream write so the Subordinate returns DBIDResp directly to the Requester. chi_dct_ctrl turns a snoop into a forwarding snoop so a peer cache supplies the data. In all three cases the Home still issues the response it alone owes, and the data never crosses it. Sections 8.1 to 8.3 trace one example of each.

The single-FSM structure is the dominant simplification in this design. The POS holds posEntries entries and detects hazards across all of them, but only one transaction advances at a time, so POS depth currently buys hazard visibility rather than throughput. Section 9.1 covers the consequences.

## 2.6 Configuration and status plane

Beside the datapath there is a narrow plane that configures it and reports on it, and it is deliberately kept out of the transaction path.

Configuration enters through one chi_cfg_slave, selected by cfgAccess == CFG_APB, which presents the APB4 register map in Section 7.1. It stages an edited region into shadow registers and commits it to every chi_sam at once when the fabric is quiescent, so no Requester can decode half of an old map and half of a new one. When cfgAccess is CFG_PARAM_ONLY the block is not built and the region table is whatever the REGION_* parameters set at elaboration; the APB pins are then tied off and a transfer attempted on them raises PSLVERR.

Status leaves through the three per-ordinal vectors already described: linkUp, linkParityError and linkCreditError. They are the only observability the design exports, and they are all link-layer facts. There is no transaction counter, no error-cause register and no interrupt.

## 2.7 RTL hierarchy

chi_interconnect_top
├── chi_cfg_check                            (elaboration assertions)
├── rnf_ports[*] / rnd_ports[*] / rni_ports[*] : chi_link_port
│   ├── chi_link_rx  x3   (REQ, RSP, DAT from the node)
│   │   └── chi_link_fsm      (receive direction activation)
│   └── chi_link_tx  x3   (RSP, SNP, DAT to the node)
│       ├── chi_lcrd_mgr      (credit pool, lcrdDepth deep)
│       └── chi_link_fsm      (transmit direction activation)
├── snf_ports[*] / sni_ports[*] : chi_link_port
│   ├── chi_link_rx  x2   (RSP, DAT from the node)
│   └── chi_link_tx  x3   (REQ, RSP, DAT to the node)
├── requester_sam[*]
│   ├── chi_sam  ── chi_hn_hash
│   └── chi_prefetch_map
├── apb_cfg : chi_cfg_slave                          [CFG_APB]
├── hnf_nodes[*] : chi_hnf_top
│   ├── chi_pos
│   ├── chi_hazard
│   ├── chi_snoop_filter
│   ├── chi_snp_gen
│   ├── chi_retry
│   ├── chi_dct_ctrl
│   ├── chi_dmt_ctrl
│   ├── chi_dwt_ctrl
│   ├── chi_stash_ctrl
│   ├── chi_atomic_alu
│   ├── chi_cmo
│   └── chi_excl_mon
├── hni_nodes[*] : chi_hni_top
│   └── chi_sam
├── mn_nodes[*] : chi_mn_dvm
└── chi_noc_top
    ├── chi_xbar ── chi_vc_arb[*]                   [CROSSBAR]
    ├── chi_ring                                    [RING]
    └── chi_mesh                                    [MESH_2D]
        └── chi_mesh_xp[*]
            └── chi_router[REQ/RSP/SNP/DAT]
                ├── chi_route_compute[*]
                └── chi_vc_arb[*]

Every link block is inside the product hierarchy; none of them is harness-only. The figures above are generated from Graphviz sources in doc-src/figures/, which make doc rebuilds.

## 2.8 Handshake discipline

Every ready/valid channel in the design obeys two rules, and the RTL is structured so that a linter can see both:

- A valid and its flit never depend combinationally on the ready they are paired with. The Home Node decides a RetryAck, a PCrdGrant and every transaction response without looking at rspOutReady.

- A module that drives both directions of a handshake writes the forward and the backward direction in separate always_comb blocks. chi_mesh uses four blocks — ingress forward, egress forward, egress ready, ingress ready — because a single block would make every crosspoint ready appear to depend on every crosspoint valid.

The consequence is that the whole fabric elaborates with no circular combinational logic, which Section 12.6 records as a passing lint result.

# Chapter 3
	PARAMETERS AND ELABORATION CONSTRAINTS

## 3.1 chi_cfg_t

All count fields are unsigned. “No explicit maximum” means the elaboration checker has no direct bound; array size, NodeID capacity and tool resources still apply. This table states what each field does; Section 3.2 states why it is a parameter at all and what choosing it wrong costs.

| Field | Default | Accepted by chi_cfg_check | Effect |
| --- | --- | --- | --- |
| numRnf | 4 | 0 or greater | External RN-F interfaces and Home sharer-vector width. |
| numRnd | 0 | 0 or greater | External RN-D interfaces. |
| numRni | 1 | 0 or greater | External RN-I interfaces. |
| numSnf | 2 | required when numHnf>0 | External SN-F interfaces and PrefetchTgt targets. |
| numSni | 1 | required when numHni>0 | External SN-I interfaces. |
| numHnf | 4 | 0 or greater | Internal HN-F endpoints and hash targets. |
| numHni | 1 | 0 or greater | Internal HN-I endpoints. |
| numMn | 1 | 0 or greater | Internal MN endpoints. |
| dataW | 256 | 128, 256 or 512 | DAT data and derived BE, tag, TU and check widths. |
| addrW | 48 | 44 through 52 | REQ address and line and tag widths. |
| nodeIdW | 11 | 7 through 16 | Active NodeID bits. Internal buses are always 16 bits wide. |
| txnIdW | 12 | 8 through 12 | Node-side TxnID width. The flit slot is always 12 bits. |
| lpidW | 5 | 1 through 5 | LPID and StashLPID overlay width. |
| rsvdcW | 0 | 0, 4, 8, 12, 16, 24 or 32 | REQ and DAT RSVDC width. |
| ccidW | 2 | 1 or 2 | Node-side CCID width. The flit slot is 2 bits. |
| groupIdW | 8 | 1 through 8 | Group-ID overlay width within the 8-bit slot. |
| mpamW | 0 | 0, 12 or 15; must be non-zero when mpamEn | MPAM bus width. |
| numIfPerNode | 1 | exactly 1 | Metadata only; multiple interfaces are rejected. |
| replChannels | 1 | exactly 1 | Metadata only; channel replication is rejected. |
| topology | MESH_2D | CROSSBAR, RING, MESH_2D | Selects the generated NoC. |
| meshX, meshY | 3, 3 | positive for mesh | Mesh coordinates and NodeID fields. |
| numDevPortsPerXp | 2 | positive for mesh | Local ports per crosspoint and NodeID field. |
| snpFilterEntries | 1024 | positive, divisible by ways | Total Home directory entries. |
| snpFilterWays | 4 | 1 through entries, exact divisor | Directory associativity. |
| posEntries | 32 | positive | POS array depth. The Home is still globally serial. |
| retryRecords | 16 | positive | Retry record table depth, that is, the number of outstanding RetryAck records the Home can hold. |
| lcrdDepth | 8 | 2 or greater | L-Credits advertised per channel and receive-buffer depth. |
| exclMonitors | 32 | positive | Exclusive monitor entries. |
| interleaveGranule | 64 | power of two, at least 64 | HN hash stripe bytes. |
| hnHashShift | 6 | no range check | XOR fold distance. A shift at or above addrW collapses the folded term to zero. |
| dataCheckEn | 1 | 0 or 1 | Adds dataW/8 DAT DataCheck bits. |
| poisonEn | 1 | 0 or 1 | Adds dataW/64 DAT poison bits. |
| parityEn | 0 | 0 or 1 | Enables flit parity generation and checking in every link. |
| mpamEn | 0 | 0 or 1 | Adds the MPAM bus to REQ and SNP. |
| mteEn | 0 | 0 or 1 | Affects DCT eligibility only. MTE semantics are absent. |
| mecEn | 0 | 0 or 1; mutually exclusive with cdaEn | Adds the 16-bit MECID to REQ, SNP and DAT. |
| pbhaEn | 0 | 0 or 1 | Adds the 4-bit PBHA field to REQ. |
| cdaEn | 0 | 0 or 1; mutually exclusive with mecEn | Adds StreamID and SecSID1 to REQ. |
| multiReqEn | 0 | 0 or 1 | Checked metadata. No multi-request behaviour. |
| dctEn | 1 | if 1, RN-F and HN-F must exist | Enables DCT eligibility and forwarding snoops. |
| dmtEn | 1 | if 1, a Home and SN-F must exist | Enables direct memory transfer. |
| dwtEn | 1 | if 1, a Home and SN-F must exist | Enables direct write transfer. |
| atomicEn | 1 | if 1, a Home must exist | Enables the atomic transaction path. |
| dvmEn | 1 | if 1, an MN and a Requester must exist | Enables the MN sequencer. |
| stashEn | 1 | if 1, RN-F and HN-F must exist | Enables stash decode, stash snoops and Data Pull. |
| exclEn | 1 | if 1, a Home and monitors must exist | Enables exclusive monitors. |
| cfgAccess | CFG_PARAM_ONLY | enum; CHI MMIO also requires HN-I | Selects no runtime writes, APB, or CHI MMIO. |

Table 3.1 chi_cfg_t fields, defaults and elaboration bounds

Because MECID and StreamID share one REQ field of width Max(E,R), only one of MEC and RME-CDA can be carried on a given interface. Elaboration rejects both being enabled.

Derived packed widths at the default configuration are REQ 153, RSP 79, SNP 106 and DAT 437 bits. The formulas are reqFlitWidth, rspFlitWidth, snpFlitWidth and datFlitWidth in chi_pkg, and top/chi_stage1_compile.sv checks each one against an independent sum of the field list so that a field added to a record cannot silently bypass the width helper.

## 3.2 Why the design is configurable

Configuration here answers two different questions, and it matters a great deal which one a given parameter answers.

Some parameters exist because CHI itself declares them configurable and both ends of a link must agree. Getting one of those wrong is an interoperability failure, and no amount of verification inside the interconnect will find it, because the mismatch is at the boundary.

The rest exist because CHI is deliberately silent about microarchitecture. Those are area, power and performance trade-offs. Getting one wrong costs throughput or area, not correctness.

A third, smaller group is accepted by the elaboration checker but carries no behaviour yet. Those are listed separately at the end so they are not mistaken for working features.

### 3.2.1 Parameters fixed by a declared CHI property

CHI Chapter B16.1, Interface properties and parameters, defines the properties an implementation declares for an interface. Every one of them must match at both ends, because flit length and field position are derived from them. There is no negotiation on the wire and no self-describing header, so a mismatch is not an error that gets reported — it is a silent misparse of every flit on that channel, and the symptom appears as corrupted addresses or impossible opcodes far from the cause.

| chi_cfg_t field | Declared CHI property | What a mismatch between the two ends produces |
| --- | --- | --- |
| dataW | B16.1.13 Data_Width | Every DAT flit is the wrong length; data, byte enables and check bits all shift. |
| addrW | B16.1.11 Req_Addr_Width | REQ and SNP lengths differ; addresses decode to the wrong region and the SAM targets the wrong node. |
| nodeIdW | B16.1.12 NodeID_Width | TgtID and SrcID misalign on all four channels; responses return to the wrong Requester. |
| rsvdcW | B16.1.28 Req_RSVDC_Width, B16.1.29 Dat_RSVDC_Width | REQ and DAT lengths differ. The field itself is user-defined, but its width is not. |
| mpamEn, mpamW | B16.1.9 MPAM_Support | REQ and SNP lengths differ. A receiver that does not expect MPAM misreads every field above it. |
| dataCheckEn | B16.1.6 Data_Check | DAT length differs, and end-to-end data integrity is silently unprotected. |
| poisonEn | B16.1.4 Data_Poison | DAT length differs, and a poisoned line is delivered as valid data. |
| parityEn | B16.1.7 Check_Type | Check signals are driven but ignored, or expected and undriven. |
| atomicEn | B16.1.1 Atomic_Transactions | A Requester issues an atomic opcode the Home cannot decode. |
| stashEn | B16.1.2 Cache_Stash_Transactions | Stash requests and stash snoops are not understood. |
| dmtEn | B16.1.3 Direct_Memory_Transfer | The Subordinate does not know it must respond directly to the Requester. |
| dctEn | B16.1.5 Direct_Cache_Transfer | A peer receives a forwarding snoop it cannot service. |
| dvmEn | B16.1.22 DVM_Support | DVM operations are issued to a node that cannot complete them. |
| mteEn | B16.1.23 MTE_Support | Memory tagging fields are misinterpreted. |
| mecEn | B16.1.25 MEC_Support, B16.1.26 MECID_Width | REQ, SNP and DAT lengths differ, and realm isolation is lost. |
| cdaEn | B16.1.16 RME_Support | StreamID and SecSID1 are absent or misplaced in REQ. |
| pbhaEn | B16.1.19 PBHA_Support | REQ length differs. |
| multiReqEn | B16.1.39 MultiReq_Support | A multi-request transaction is treated as a single request. |
| retryRecords | B16.1.38 Retry_Support, with B16.1.34 Num_RP_REQ and B16.1.35 Shared_Credits_REQ | The P-Credit pool the Requester believes exists does not match what the Home can track. |

Table 3.2 Configuration fields fixed by a declared CHI interface property

dwtEn has no row, and the reason is instructive. There is no declared property for Direct Write Transfer. It is requested per transaction through the DoDWT field of the REQ flit (B13.10.24), so a Home that does not implement it simply never sets the bit and no agreement is needed in advance. That is exactly why dmtEn mirrors a B16.1 property while dwtEn is a purely local choice.

Because these properties set flit length, each one has a measurable price. The following are exact deltas against the default configuration, whose packed widths are REQ 153, RSP 79, SNP 106 and DAT 437 bits:

| Change from the default | REQ | RSP | SNP | DAT | Total |
| --- | --- | --- | --- | --- | --- |
| nodeIdW 11 to 16 | +15 | +10 | +10 | +15 | +50 |
| nodeIdW 11 to 7 | -12 | -8 | -8 | -12 | -40 |
| addrW 48 to 52 | +4 | 0 | +4 | 0 | +8 |
| dataW 256 to 512 | 0 | 0 | 0 | +334 | +334 |
| dataW 256 to 128 | 0 | 0 | 0 | -167 | -167 |
| rsvdcW 0 to 32 | +32 | 0 | 0 | +32 | +64 |
| mpamEn with mpamW 12 | +12 | 0 | +12 | 0 | +24 |
| mpamEn with mpamW 15 | +15 | 0 | +15 | 0 | +30 |
| mecEn | +16 | 0 | +16 | +16 | +48 |
| cdaEn | +17 | 0 | 0 | 0 | +17 |
| pbhaEn | +4 | 0 | 0 | 0 | +4 |
| dataCheckEn off | 0 | 0 | 0 | -32 | -32 |
| poisonEn off | 0 | 0 | 0 | -4 | -4 |

Table 3.3 Flit width cost of moving each parameter off its default

Those totals are per flit, and every flit width is replicated across every port, every crosspoint and every receive buffer, so the cost multiplies by the fabric size. nodeIdW is the parameter to watch: it appears three times in REQ, twice in RSP, twice in SNP and three times in DAT, so a width chosen for future expansion is paid for on all four channels at once. The default of 11 is the narrowest value that still encodes the default mesh placement, as Section 3.2.3 explains.

parityEn behaves differently from the rest of the table: it does not change flit length, it adds a separate check bus alongside each channel, one bit per eight payload bits. At the default widths that is 20, 10, 14 and 55 wires for REQ, RSP, SNP and DAT respectively, per direction, per port.

### 3.2.2 Parameters the specification leaves to the implementation

For this group CHI states a requirement and then explicitly declines to constrain how it is met. B3.1 does this for the address map — “The exact format and structure of the SAM is IMPLEMENTATION DEFINED and is outside the scope of this specification” — and B3.2 does it for node identity: “Defining and assigning a node ID for each node in a system is IMPLEMENTATION DEFINED”. Everything below is configurable because the specification left the choice open, not because interoperability demands it.

| Field | Why it is a parameter | What you trade |
| --- | --- | --- |
| numRnf, numRnd, numRni, numSnf, numSni | The system population is not a protocol property. | Port count, and for numRnf the Home sharer-vector width and snoop walk length. |
| numHnf, numHni, numMn | Home count sets coherent throughput and interleaving. | More Homes means more parallelism and more directory area. |
| topology | CHI specifies no transport. Any network that delivers flits is legal. | See Section 5: crossbar for latency, ring for area, mesh for scale. |
| meshX, meshY, numDevPortsPerXp | Mesh shape is microarchitecture. | Hop count against crosspoint count; also consumes NodeID bits. |
| snpFilterEntries, snpFilterWays | CHI requires a Home to track sharers but not how. | Directory area against back-invalidation traffic. See Section 3.2.3. |
| posEntries | Transaction tracking depth is unspecified. | Hazard visibility. It does not currently buy throughput. |
| retryRecords | The Home decides how many retried Requesters it will remember. | Retry table area against how often the Home must back-pressure instead of retrying. |
| lcrdDepth | CHI fixes the credit protocol, not the pool size. | Receive-buffer area against sustainable throughput. See Section 3.2.3. |
| exclMonitors | Monitor count is unspecified; the architecture permits spurious failure. | Monitor area against the spurious exclusive-failure rate. |
| interleaveGranule, hnHashShift | Address-to-Home mapping is part of the SAM, so it is implementation defined. | Stripe size against locality and hot-spotting. |
| txnIdW | The flit slot is a fixed 12 bits; how many a node uses is its own business. | Outstanding transactions per Requester against tracking area. |
| lpidW, ccidW, groupIdW | These occupy fixed flit slots. The parameter sets how many bits are significant. | Only internal comparator and storage width. Flit length is unaffected. |
| cfgAccess | CHI defines no register map, as Section 7 explains. | No runtime writes, a second bus protocol, or CHI-MMIO traffic. |
| NUM_REGIONS | SAM depth follows the system memory map. | Region table area against map flexibility. |
| AGE_LIMIT | Arbitration policy is unspecified. | Starvation bound against fairness overhead. |
| NODE_IDS, PLACEMENTS | B3.2 assigns this to the implementation. | Physical placement against route length. Must be collision-free. |

Table 3.4 Configuration fields the specification leaves implementation defined

### 3.2.3 Choosing the resource depths

Four of these parameters need real reasoning rather than a default, and two of the defaults are deliberately unrealistic.

lcrdDepth is set by the round trip, not by burst size. A credit is returned only after the receiver pops the flit, so the sustainable rate is lcrdDepth flits per credit round-trip latency. If the round trip is eight cycles, lcrdDepth of 8 sustains one flit per cycle and anything less stalls the channel even when the receiver is completely idle. This is why the elaboration checker enforces a minimum of 2 rather than 1: a single credit serialises the channel to one flit per round trip. The default of 8 matches the internal crosspoint latency of the default mesh.

snpFilterEntries must cover the aggregate of the caches it tracks. A directory smaller than the sum of the RN-F caches cannot hold every live line, so it evicts lines that are still cached, and every such eviction becomes a back-invalidation snoop — real traffic caused purely by directory capacity. The default of 1024 entries across 4 ways, tracking 4 RN-F, is sized for simulation speed and is far too small for a real system; four Requesters with even a 512-line cache each need 2048 entries before capacity back-invalidations stop. Size this from the actual cache sizes, not from the default.

posEntries does not currently buy throughput. The Home has one globally serial FSM, so although the POS holds posEntries entries and chi_hazard checks a new request against all of them, only one transaction advances at a time. Increasing it improves hazard visibility and nothing else. It becomes a throughput parameter only once the FSM is pipelined, which Section 9.1 lists as a known limitation.

retryRecords bounds how many Requesters can be waiting for a P-Credit. When the table is full this design back-pressures the request rather than issuing another RetryAck, which keeps credit accounting exact at the cost of holding the channel. Section 9.3 covers why that choice was made. Sizing it below the number of Requesters guarantees back-pressure under load.

nodeIdW has a floor set by the mesh geometry, not by the node count. The NodeID encodes the routing fields, so deviceIdWidth = nodeIdW - clog2(meshY) - clog2(meshX) - clog2(numDevPortsPerXp) must stay positive. At the default 3 by 3 mesh with 2 device ports that is 11 - 2 - 2 - 1 = 6 bits left for the device identifier. Widening the mesh therefore consumes NodeID bits, and elaboration rejects a combination where nothing is left. This is the one parameter in Section 3.2.1 whose value is constrained by a Section 3.2.2 choice, which is why it cannot simply be set to the minimum the node count would allow.

### 3.2.4 Parameters accepted but not yet honoured

These are validated by chi_cfg_check so that an illegal value is rejected early, but no RTL consumes them as behaviour. They are configuration surface reserved for later work, not features.

| Field | Current status |
| --- | --- |
| numIfPerNode | Must be exactly 1. Multiple interfaces per node are rejected, consistent with the single-chi_if decision in Section 4.1. |
| replChannels | Must be exactly 1. Channel replication is rejected. |
| multiReqEn | Checked only. MultiReq and CacheLineID are carried in the flit but no multi-request behaviour exists. |
| mteEn | Affects DCT eligibility only. No MTE tag semantics are implemented. |
| pbhaEn | Adds the field to REQ. The attributes are carried, never acted on. |
| mpamEn, mpamW | The MPAM bus is carried end to end but no partition is enforced. |

Table 3.5 Configuration fields accepted but not yet honoured

## 3.3 Top parameters and illegal combinations

| Parameter | Default and legal rule | Effect |
| --- | --- | --- |
| AGE_LIMIT | 16; positive | QoS arbiter starvation promotion threshold. |
| NUM_REGIONS | 4; positive | SAM table depth and select width. |
| NUM_NODES | sum of all node counts; must equal that sum | Fabric endpoint count and the index space for the link status outputs. |
| NODE_IDS | all-ones sentinel derives the default; effective IDs must be unique | Endpoint identities and routing targets. |
| USE_DEFAULT_PLACEMENTS | 1 | Selects the fixed 14-entry map; requires exactly 14 nodes. |
| PLACEMENTS | each valid, correct class and index, in range, collision-free | Custom mesh and local-port placement. |
| CFG_MMIO_BASE | top 4 KiB of the address space | Must be 4 KiB aligned in CHI-MMIO mode. |
| USE_DEFAULT_REGIONS | 1 | Selects the default map in Section 6. |
| REGION_* | arrays of NUM_REGIONS | Custom base, inclusive limit, target and attributes. |

Table 3.6 Top-level parameters and the combinations refused at elaboration

For the mesh, meshX*meshY*numDevPortsPerXp >= NUM_NODES, deviceIdWidth(CFG) > 0, placement coordinates and ports must fit, NodeIDs must encode the placement routing fields, and no NodeID or placement may collide. Default placement is valid only for the default 14-node population. HN-F to SN-F and HN-I to SN-I striping uses modulo, so the nonzero subordinate checks are essential.

## 3.4 Top-level ports beyond the interfaces

| Port | Direction | Width | Purpose |
| --- | --- | --- | --- |
| clk, resetn | in | 1 | Single clock and synchronous active-low reset. |
| PADDR, PSEL, PENABLE, PWRITE, PWDATA, PSTRB, PPROT | in | 12, 1, 1, 1, 32, 4, 3 | APB4 configuration request. |
| PRDATA, PREADY, PSLVERR | out | 32, 1, 1 | APB4 configuration response. |
| linkUp | out | NUM_NODES | Set while any receive channel of that node’s link is in RUN. |
| linkParityError | out | NUM_NODES | Flit parity mismatch observed on that node’s link. |
| linkCreditError | out | NUM_NODES | Receive-buffer overflow or L-Credit overflow or underflow on that node’s link. |

Table 3.7 Top-level ports outside the node interfaces

Bits of the three status vectors that correspond to the internal HN-F, HN-I and MN ordinals read zero, because those endpoints have no external link.

# Chapter 4
	THE SINGLE chi_if NODE-FACING INTERFACE

## 4.1 Superset rule

There is exactly one SystemVerilog interface declaration in the repository: if/chi_if.sv. Every external RN-F, RN-D, RN-I, SN-F and SN-I port is an array of this superset interface. There are no class-specific interfaces and no separate internal transport interface.

The interface has no modports, so the directions below are integration rules rather than compiler-enforced directions. “Node” means the external endpoint and “IC” means chi_interconnect_top. A logical count of zero still creates one sentinel interface, because portable SystemVerilog cannot declare a zero-length interface array. That sentinel is inactive and must be tied off.

## 4.2 Signal widths and drivers

Let RW=reqFlitWidth(CFG), SW=rspFlitWidth(CFG), NW=snpFlitWidth(CFG), DW=datFlitWidth(CFG) and PW(W)=ceil(W/8).

| Signal group | Width | Driver | Consumer | Applicability |
| --- | --- | --- | --- | --- |
| CLK, RESETn | 1 each | IC | Node | All classes; mirrors clk and the synchronous active-low resetn. |
| TXLINKACTIVEREQ | 1 | Node | IC | All classes; drives the node-to-IC link FSM out of STOP. |
| TXLINKACTIVEACK | 1 | IC | Node | All classes; asserted once the IC receivers have credits to advertise. |
| RXLINKACTIVEREQ | 1 | IC | Node | All classes; the IC activates its transmit links while out of reset. |
| RXLINKACTIVEACK | 1 | Node | IC | All classes; gates IC credit acceptance and flit transmission. |
| TXSACTIVE, RXSACTIVE | 1 each | IC | Node | All classes; follow reset. |
| SYSCOREQ | 1 | Node | IC | All classes. |
| SYSCOACK | 1 | IC | Node | All classes; echoes SYSCOREQ. |
| TXREQFLITPEND, TXREQFLITV, TXREQFLIT[RW-1:0] | 1, 1, RW | Node | IC | RN-F, RN-D, RN-I. PEND is accepted and not used for early arbitration. |
| TXREQLCRDV | 1 | IC | Node | Requesters; one pulse per REQ L-Credit granted by the IC receiver. |
| RXREQFLITPEND, RXREQFLITV, RXREQFLIT[RW-1:0] | 1, 1, RW | IC | Node | SN-F, SN-I. |
| RXREQLCRDV | 1 | Node | IC | SN-F, SN-I; one pulse per REQ L-Credit granted to the IC transmitter. |
| TXRSPFLITPEND, TXRSPFLITV, TXRSPFLIT[SW-1:0] | 1, 1, SW | Node | IC | All five classes. |
| TXRSPLCRDV | 1 | IC | Node | All classes; RSP L-Credit pulse. |
| RXRSPFLITPEND, RXRSPFLITV, RXRSPFLIT[SW-1:0] | 1, 1, SW | IC | Node | All five classes. |
| RXRSPLCRDV | 1 | Node | IC | All classes; RSP L-Credit pulse. |
| TXSNPFLITPEND, TXSNPFLITV, TXSNPFLIT[NW-1:0] | 1, 1, NW | Node | IC | Unused for every external class; no port in this design receives snoops from a node. |
| TXSNPLCRDV | 1 | IC | Node | Tied low for every class. |
| RXSNPFLITPEND, RXSNPFLITV, RXSNPFLIT[NW-1:0] | 1, 1, NW | IC | Node | RN-F, RN-D, RN-I; tied inactive for SN-F and SN-I. |
| RXSNPLCRDV | 1 | Node | IC | Requesters; SNP L-Credit pulse. |
| TXDATFLITPEND, TXDATFLITV, TXDATFLIT[DW-1:0] | 1, 1, DW | Node | IC | All five classes. |
| TXDATLCRDV | 1 | IC | Node | All classes; DAT L-Credit pulse. |
| RXDATFLITPEND, RXDATFLITV, RXDATFLIT[DW-1:0] | 1, 1, DW | IC | Node | All five classes. |
| RXDATLCRDV | 1 | Node | IC | All classes; DAT L-Credit pulse. |
| TXREQFLITCHK, RXREQFLITCHK | PW(RW) | Same side as the flit | Opposite side | Live per-byte flit parity when parityEn is set. |
| TXRSPFLITCHK, RXRSPFLITCHK | PW(SW) | Same side as the flit | Opposite side | Live per-byte flit parity when parityEn is set. |
| TXSNPFLITCHK, RXSNPFLITCHK | PW(NW) | Same side as the flit | Opposite side | Live per-byte flit parity when parityEn is set. |
| TXDATFLITCHK, RXDATFLITCHK | PW(DW) | Same side as the flit | Opposite side | Live per-byte flit parity when parityEn is set. |
| scalar *CHK beside control, valid and credit signals | 1 each | Same side as the checked base signal | Opposite side | Declared for all classes. Node-driven checks are ignored and IC-driven checks are zero. |

Table 4.1 chi_if signal groups, widths, drivers and applicability

Unused node outputs must be tied to zero. Unused IC outputs are already driven inactive. Integrators must not infer legal channel capability from the presence of a signal in this superset.

## 4.3 Link credit behaviour

Each channel in each direction has its own credit counter and its own activation FSM. On leaving STOP, a receiver advertises lcrdDepth credits one LCRDV pulse at a time and refills a credit every time a buffered flit is consumed by the fabric. A transmitter sends a flit only while in RUN and holding at least one credit. Deactivation waits until the channel’s credits are quiescent, which is why the port ORs the per-channel activation signals: the OR holds the handshake asserted until the last channel has drained.

Receive-buffer overflow, credit overflow and credit underflow are detected and reported on linkCreditError; they indicate a peer that violated the credit contract, not a recoverable condition.

# Chapter 5
	TOPOLOGIES, ROUTING, ARBITRATION AND PROGRESS

## 5.1 Common transport

chi_noc_flit_t carries the channel, QoS, target NodeID and an opaque maximum-size DAT payload. REQ, RSP, SNP and DAT are separate ready/valid fabrics that share no buffers and no arbiters. Backpressure requires valid and payload stability. These are internal ready/valid channels; the CHI L-Credit channels terminate in chi_link_port.

## 5.2 Crossbar

Each output selects the highest-QoS request. Equal QoS starts at a rotating index. Once a request’s age reaches AGE_LIMIT, aged requests exclude non-aged requests. A stalled grant locks until it is accepted. An uncongested output accepts one flit per channel per cycle. Unknown target IDs remain backpressured.

## 5.3 Bidirectional ring

Distance is computed from endpoint array order. The shorter direction wins and an exact tie goes clockwise. Each node, direction and channel has one registered slot. When local injection and transit contend for an empty slot, a preference bit alternates after the winner, which prevents permanent local or transit starvation under continued service. Clockwise, counter-clockwise and local ejection requests use a three-way rotating priority. The ring has no QoS or age threshold.

For planning, use a conservative sustained rate of 0.5 flit per cycle per directional link per channel under injection and transit contention. That is a budgeting bound, not a guarantee: uncongested trains do better and blocked ejection does worse.

## 5.4 Mesh, placement and NodeID

Routing is deterministic XY: east or west to the destination X, then north or south to the destination Y, then the local device port. Coordinates start at the south-west corner (0,0).

north
        x=0                    x=1                    x=2
y=2  [p0 HNF2, p1 --]     [p0 --,   p1 --]      [p0 HNF3, p1 --]
y=1  [p0 SNI0, p1 RNF2]---[p0 MN0,  p1 HNI0]----[p0 SNF1, p1 RNF3]
y=0  [p0 HNF0, p1 RNF0]---[p0 SNF0, p1 RNF1]----[p0 HNF1, p1 RNI0]
                         south

The project-defined NodeID encoding is:

NodeID     = {meshY, meshX, devicePort, deviceId}
field width = max(1, ceil(log2(cardinality)))   for Y, X and port
deviceId width = nodeIdW - Ywidth - Xwidth - portwidth

At the defaults this gives [10:9]=Y, [8:7]=X, [6]=port and [5:0]=ordinal. Ordinals run RN-F, RN-D, RN-I, SN-F, SN-I, HN-F, HN-I, MN. The default HN-F NodeIDs are HNF0 11'h008, HNF1 11'h109, HNF2 11'h40a and HNF3 11'h50b. The hash array follows ordinal order and the corner placement supplies the spatial distribution.

Each chi_router contributes one registered crosspoint output stage, and the wires between crosspoints are combinational. For source (sx,sy) and destination (dx,dy):

hops = |dx-sx| + |dy-sy|
planning latency ~ (hops + 1) crosspoint cycles + endpoint and link cycles

Stalls add arbitration and destination-backpressure cycles. The link layer adds one receive-buffer cycle in each direction.

## 5.5 Snoop distribution, deadlock and livelock

Home snoops are sequential unicasts in ascending sharer index, and only one snoop is outstanding from the generator, so latency sums the round trips of all sharers. HN-F i is statically striped to SN-F i % numSnf, and the HN-I uses the same rule for SN-I. PrefetchTgt does not follow that striping: it uses its own address-to-SN-F mapper.

Separate channel fabrics prevent one protocol channel from consuming another channel’s transport buffer, and mesh XY routing is dimension-ordered. Those choices remove routing-dependency cycles, but there is no complete protocol-level deadlock proof in this repository. Ring finite slots plus blocked ejection can backpressure a direction globally. Crossbar and mesh age arbitration and the ring’s rotating preferences address local starvation, but there is no end-to-end livelock watchdog and no bounded-completion proof.

# Chapter 6
	SYSTEM ADDRESS MAP

## 6.1 Lookup and default regions

Regions use inclusive [base, limit] bounds with ascending-index first-match priority. Attributes are [3:2]=kind, [1]=secureOnly, [0]=valid. A secure-only match by a non-secure request returns unmappedError; the lookup does not fall through to an overlapping lower-priority region.

With NUM_REGIONS=4 and USE_DEFAULT_REGIONS=1:

| Index | Base | Inclusive limit | Kind and target | Valid |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2^(addrW-1)-1 | HN-F hash | 1 |
| 1 | 2^(addrW-1) | CFG_MMIO_BASE-1 | HNI0 when present | 1 |
| 2 | CFG_MMIO_BASE | 2^addrW-1 | MMIO, HNI0 when present | 1 |
| 3 | 0 | 0 | zero target | 0 |

Table 6.1 Default region table loaded at reset

The default CFG_MMIO_BASE is 2^addrW - 4096.

## 6.2 HN hash and worked example

granuleNumber = address >> log2(interleaveGranule)
hashValue     = granuleNumber xor (granuleNumber >> hnHashShift)
hnfIndex      = hashValue mod numHnf
target        = HN_NODE_IDS[hnfIndex]

At the defaults, address 0x0000_0000_10c0 gives granule number 0x43, folded value 0x42, index 2 and target HNF2 (11'h40a).

## 6.3 PrefetchTgt mapping

PrefetchTgt is sent by a Requester directly to a Subordinate and never reaches the Home, so it cannot use the HN hash. chi_prefetch_map maps the address to an SN-F with the same interleave granule:

granuleNumber = address >> log2(interleaveGranule)
snfIndex      = granuleNumber mod numSnf
target        = SNF_NODE_IDS[snfIndex]

The transaction has no response, so nothing is tracked for it, and it must never be given a RetryAck. Both Home types accept and discard it if one reaches them.

## 6.4 Retargeting

APB or CHI-MMIO updates replace one active region atomically at the SAM clock edge, and the update is broadcast to the requester SAMs and to the HN-I staging and SAM state. APB requires the top’s trafficQuiescent, which checks external fabric ingress valids and Home and MN idle, but not every internal topology slot. CHI-MMIO commit has no quiescence check. Software must avoid overlapping active traffic and must stage base, limit, target and attributes before commit. The RTL does not validate base <= limit or target existence on a runtime update.

# Chapter 7
	CONFIGURATION ACCESS

CHI defines no register map for this project. The access mechanism, offsets, security policy and commit protocol in this section are implementation defined.

## 7.1 APB4 map

APB runs on clk and PREADY is always 1. Addresses are 12-bit and word aligned. Writes merge the bytes selected by PSTRB. Reads are unrestricted. A write with PPROT[1]=1 is rejected as non-secure. Unsupported or misaligned accesses, out-of-range SELECT writes and non-quiescent COMMIT writes return PSLVERR.

| Offset | Name | Access | Reset or read value | Write effect |
| --- | --- | --- | --- | --- |
| 0x000 | SELECT | RW | 0 | Select a region; reject a value at or above NUM_REGIONS. |
| 0x004 | BASE_LO | RW | REGION_BASE[n][31:0] | Stage the low address bytes. |
| 0x008 | BASE_HI | RW | upper configured address bits | Stage bits addrW-1:32. |
| 0x00c | LIMIT_LO | RW | REGION_LIMIT[n][31:0] | Stage the low limit bytes. |
| 0x010 | LIMIT_HI | RW | upper configured limit bits | Stage bits addrW-1:32. |
| 0x014 | TARGET | RW | REGION_TARGET[n] | Stage the target NodeID. |
| 0x018 | ATTRIBUTES | RW | REGION_ATTRIBUTES[n] | Stage the low four bits. |
| 0x01c | COMMIT | W, reads zero | 0 | If byte strobe 0 is set and traffic is quiescent, pulse one region update. |

Table 7.1 APB4 register map of chi_cfg_slave

The required software order is SELECT, stage all changed fields, stop initiators, observe system-specific quiescence, then COMMIT. A COMMIT with PSTRB[0]=0 does nothing and does not error.

## 7.2 CHI MMIO map and limitations

The HN-I recognizes any address in the 4 KiB window at CFG_MMIO_BASE.

| Low-byte offset | Read and write object |
| --- | --- |
| 0x00 | selected region |
| 0x08 | full staged base |
| 0x10 | full staged inclusive limit |
| 0x18 | staged target |
| 0x20 | staged attributes |
| 0x28 | a write commits; a read falls through to the selected-region value |

Table 7.2 CHI MMIO offsets recognised by the HN-I

A read returns one CompData; a write consumes the next DAT flit and returns Comp. The path does not check the DAT opcode, byte enables, security or PAS, privilege, base and limit validity, target validity, or quiescence. Unsupported offsets silently read SELECT or ignore the write and still complete successfully, and an invalid SELECT payload is silently ignored. Only one HN-I transaction is active at a time. This is a reference control path, not a hardened management interface.

# Chapter 8
	TRANSACTION FLOWS

Each flow below is the architecturally intended sequence. The note after each table states what the RTL does.

## 8.1 ReadShared with DCT

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | Requesting RN-F | HN-F | ReadShared |
| 2 | HN-F | Owning RN-F | SnpSharedFwd |
| 3 | Owning RN-F | Requesting RN-F | CompData with the forwarded line |
| 4 | Owning RN-F | HN-F | SnpRespFwded |
| 5 | HN-F | Requesting RN-F | completion as required |

Table 8.1 ReadShared served by direct cache transfer

RTL status: Partial. Eligibility, the forwarding snoop and SnpRespFwded collection exist. The fallback when the forwarding peer cannot supply data is not implemented.

## 8.2 ReadUnique with DMT

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | RN-F | HN-F | ReadUnique |
| 2 | HN-F | RN-F sharers | snoops as required by the directory |
| 3 | HN-F | SN-F | ReadNoSnp with ReturnNID and ReturnTxnID naming the Requester |
| 4 | SN-F | RN-F | CompData direct to the Requester |
| 5 | HN-F | RN-F | the response the Home still owes, when one is required |

Table 8.2 ReadUnique served by direct memory transfer

RTL status: Implemented. chi_dmt_ctrl decides eligibility, the downstream request carries the Requester’s identity and a Home-unique TxnID, and the Home issues only its own remaining response. ReadNoSnpSep is handled on the same path.

## 8.3 WriteBackFull with DWT

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | RN-F | HN-F | WriteBackFull |
| 2 | HN-F | SN-F | forwarded write with DoDWT set |
| 3 | SN-F | RN-F | DBIDResp direct to the Requester |
| 4 | RN-F | SN-F | CopyBackWrData via the Home |
| 5 | SN-F | HN-F | Comp |
| 6 | HN-F | RN-F | forwarded completion |

Table 8.3 WriteBackFull served by direct write transfer

RTL status: Implemented for the direct-write path. Without DWT the Home combines Comp into CompDBIDResp itself. Partial-write byte-enable rules and write cancellation remain incomplete.

## 8.4 CleanUnique with snoops

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | RN-F | HN-F | CleanUnique |
| 2 | HN-F | Sharer A | selected snoop |
| 3 | Sharer A | HN-F | SnpResp |
| 4 | HN-F | Sharer B | selected snoop |
| 5 | Sharer B | HN-F | SnpResp |
| 6 | HN-F | SN-F | downstream request when required |
| 7 | HN-F | RN-F | completion |

Table 8.4 CleanUnique with sequential snoops

RTL status: Partial and sequential. The opcode is recognized, but snoop selection and directory transitions are simplified.

## 8.5 AtomicLoad

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | Requester | HN-F | AtomicLoad with the operation in the opcode |
| 2 | HN-F | Requester | DBIDResp |
| 3 | Requester | HN-F | NonCopyBackWrData carrying the operand |
| 4 | HN-F | RN-F sharers | snoops when the line is shared, which may supply the initial value |
| 5 | HN-F | SN-F | ReadNoSnp for the initial value when no snoop supplied it |
| 6 | SN-F | HN-F | CompData with the initial value |
| 7 | HN-F | SN-F | WriteNoSnpFull with the computed value |
| 8 | HN-F | Requester | CompData with the original value, or Comp for a store |

Table 8.5 AtomicLoad executed at the Home

RTL status: Implemented. chi_atomic_alu computes AtomicStore, AtomicLoad, AtomicSwap and AtomicCompare with both operand endiannesses. Big-endian operands are byte-reversed within the operand size before the arithmetic and reversed again on the way out, because atomic data is byte invariant. A snoop that invalidates a dirty peer copy supplies the initial value and removes the downstream read. The datapath is 64 bits wide, so operands wider than 8 bytes are unsupported. The operation is always performed at the Home; there is no far-atomic negotiation.

## 8.6 Request Retry

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | Requester | HN-F | request with AllowRetry=1 |
| 2 | HN-F | Requester | RetryAck carrying the Home’s chosen PCrdType |
| 3 | HN-F | Requester | PCrdGrant with the same PCrdType once a resource exists |
| 4 | Requester | HN-F | the request re-sent with AllowRetry=0 and the granted PCrdType |
| 5 | Requester | HN-F | PCrdReturn instead of step 4 if the credit is no longer needed |

Table 8.6 Request Retry followed by a P-Credit grant

RTL status: Implemented. The Home records the SrcID, QoS and credit class behind each RetryAck in a retryRecords-deep table, and the record is taken when the RetryAck is accepted rather than when it is first presented, so a back-pressured response channel cannot book the same Requester twice. QoS selects which record is granted next and age breaks ties. PCrdGrant names the recorded Requester and carries no TxnID and no DBID. When the record table is full, the request is back-pressured instead of retried, which keeps the credit accounting exact.

## 8.7 WriteUniqueFullStash with Data Pull

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | RN-I | HN-F | WriteUniqueFullStash with StashNID and optional StashLPID |
| 2 | HN-F | RN-I | DBIDResp |
| 3 | RN-I | HN-F | NonCopyBackWrData |
| 4 | HN-F | Stash target RN-F | SnpMakeInvalidStash or SnpUniqueStash |
| 5 | Stash target RN-F | HN-F | SnpResp with DataPull set, or without it |
| 6 | HN-F | SN-F | the read implied by the Data Pull |
| 7 | SN-F | HN-F | CompData |
| 8 | HN-F | Stash target RN-F | CompData completing the implied read |
| 9 | HN-F | RN-I | Comp |

Table 8.7 WriteUniqueFullStash with a Data Pull

RTL status: Implemented. chi_stash_ctrl chooses the snoop from the stash opcode and whether the line is fully written, resolves the target from StashNID and StashNIDValid, and passes StashLPID and StashLPIDValid through the snoop’s 12-bit slot. An accepted Data Pull is turned into ReadUnique or ReadNotSharedDirty and the directory records the stash target as the new owner. The independent StashOnce forms return StashDone, and StashOnceSep forms return the combined CompStashDone.

## 8.8 DVM Sync

| Step | From | To | Message |
| --- | --- | --- | --- |
| 1 | Initiating Requester | MN | DVMOp |
| 2 | MN | Requester A | SnpDVMOp |
| 3 | Requester A | MN | SnpResp |
| 4 | MN | Requester B | SnpDVMOp |
| 5 | Requester B | MN | SnpResp |
| 6 | MN | Initiating Requester | Comp |

Table 8.8 DVM Sync distributed by the MN

RTL status: Partial. Sequential broadcast and response collection exist. The two-part DVM payload is not decoded, so no operation-specific ordering is applied.

# Chapter 9
	HOME NODE INTERNALS

## 9.1 Serialization

chi_pos holds posEntries records with the line address, requester, TxnID, opcode and completion flags. chi_hazard blocks same-line overlap, blocks an ordered request while any POS entry is active, and blocks all new requests while an ordered entry is active. That is more conservative than a line-only POS.

The controlling FSM in chi_hnf_top is the stronger limitation. It accepts a normal request only in HNF_IDLE, stores one activeReq, and does not return to idle until completion or CompAck. Therefore all normal Home transactions are globally serialized regardless of posEntries, and the multi-entry POS does not create concurrency. The FSM has 22 states: the baseline request, snoop, response and back-invalidation states; eight atomic states; an ordered-receipt state; a DMT response state; two stash completion states; and two Data Pull states.

Two requests bypass the FSM entirely. PCrdReturn carries no transaction and PrefetchTgt has no response and must never be retried, so both are accepted and discarded without allocating a POS entry.

## 9.2 Directory

The snoop filter is set-associative, tracks precise RN-F sharers and one owner, uses rotating replacement ways, and requests sequential back-invalidation before replacing a valid full set. Updates are simplified: a write invalidates, a unique read makes the requester the sole sharer and owner, and another recognized coherent read ORs the requester into the sharer vector. After an accepted Data Pull the stash target becomes the owner. This is not the complete Issue H state machine.

## 9.3 Retry credits

chi_retry is the complete credit loop. On a RetryAck it records the SrcID, the QoS and the credit class derived from the request category, and it reports the per-class pending count and the per-class outstanding-credit count. CHI_CREDIT_TYPES is 4 of the 16 PCrdType values Issue H allows; the Home partitions its resources by request category. A grant is issued only when the Home is idle, the POS can allocate, no back-invalidation is running, and no previously granted credit is still outstanding, which prevents the Home from promising the same resource twice. A credit leaves the accounting when a request arrives with AllowRetry deasserted or when the Requester returns it with PCrdReturn.

## 9.4 Ordering

A request that asks for a read receipt is answered with ReadReceipt from the HNF_ORDER_RECEIPT state before the data path continues. Ordered requests are globally serialized by the hazard logic. Endpoint order and request order are not distinguished, so the implementation is stricter than Issue H requires.

# Chapter 10
	RN-F BOUNDARY CACHE-STATE MODEL

These tables are the required architectural model for an RN-F cache controller that connects to this interconnect. They are not behaviour implemented here. chi_pkg defines the I, SC, UC, UD, SD and UDP encodings, but there is no RN line-state storage and no transition controller in this repository.

## 10.1 Request-driven target states

| RN starting state | Operation | Intended successful target | Required external effects |
| --- | --- | --- | --- |
| I | ReadShared or ReadClean | SC, or a permitted dirty shared state from the response | Install the returned line and honour the response state. |
| I or SC | ReadUnique or MakeUnique | UC or UD according to the returned dirtiness | Other sharers must be invalidated before exclusive ownership. |
| SC | CleanUnique | UC | Complete snoops and cleaning before uniqueness. |
| UC | local write | UD | Mark dirty locally. |
| UD or SD | WriteBackFull | I, or a clean retained state according to the result | Transfer the full dirty line and wait for the required completion. |
| Any valid | Evict or MakeInvalid | I | Satisfy writeback and invalidation obligations first. |
| Any | atomic | the state dictated by the atomic response | Serialize the operand, the data update and the returned value. |
| Any | stash snoop target | the state the stash snoop requires | Optionally request a Data Pull and complete the implied read. |

Table 10.1 Required RN-F target state for each request

## 10.2 Snoop-driven transitions

| Starting state | Snoop class | Intended result | Data obligation |
| --- | --- | --- | --- |
| I | any | I | Respond no copy; no data. |
| SC | shared or clean | SC | Usually no dirty data. |
| SC | unique or make-invalid | I | Invalidate and respond. |
| UC | shared | SC | Downgrade ownership. |
| UC | unique or make-invalid | I | Invalidate. |
| UD | shared | SD or SC as the response requires | Supply dirty data or transfer responsibility. |
| UD, SD or UDP | unique or make-invalid | I | Supply dirty or partial data as required, then invalidate. |
| Valid owner | forwarding snoop | the result dictated by the forward response | Send data to the forward target and the response to the Home. |
| Any | stash snoop | the state the stash opcode requires | Optionally set DataPull in the response to request the line. |

Table 10.2 Required RN-F transition for each snoop class

Partial and empty line handling, UDP rules, MTE allocation tags, poison and DataCheck propagation, and the complete set of legal response-state combinations remain the RN implementer’s responsibility.

# Chapter 11
	CLOCKING, RESET, CDC AND PERFORMANCE

All RTL uses one clk. APB PCLK is connected to it and every external chi_if.CLK mirrors it. There are no asynchronous clock domains, synchronizers or CDC FIFOs. Multi-clock integration requires external adapters and CDC analysis.

resetn and PRESETn are active low but sampled synchronously, because every sequential block uses always_ff @(posedge clk) and tests if (!resetn). There is no asynchronous reset edge. Integration must assert reset long enough for it to be sampled and must deassert it synchronously. The interconnect activates its own transmit links while out of reset; sequenced link power-down is left to integration.

Reference throughput limits:

- Crossbar: at most one accepted flit per output, per channel, per cycle.

- Ring: one slot per node, direction and channel; use the conservative 0.5 flit per cycle planning rate from Section 5.3.

- Mesh: one registered slot per crosspoint output and channel; no-contention network latency is approximately hops+1 crosspoint cycles.

- Link: one receive-buffer cycle per direction, and a channel stalls whenever its credit count reaches zero, so lcrdDepth bounds the round-trip bandwidth of each channel.

- The Home, the HN-I and the MN each process one architectural transaction at a time, and sequential snoops and DVMs add one full response round trip per target.

No timing constraints, synthesis reports, PPA results, latency measurements or performance guarantees are supplied.

# Chapter 12
	VERIFICATION, SOURCE MANIFEST AND LINT

## 12.1 Bound SVA inventory

| File | Assertions | Covers |
| --- | --- | --- |
| chi_link_sva.sv | NO_SEND_WITHOUT_RUN_OR_CREDIT, NO_CREDIT_OVERFLOW, NO_CREDIT_UNDERFLOW, STOP_TRANSITION_VALID, ACTIVATE_TRANSITION_VALID, RUN_TRANSITION_VALID, DEACTIVATE_TRANSITION_VALID, DEACTIVATION_CREDITS_DRAINED | LINK_ACTIVATION, LINK_DEACTIVATION |
| chi_link_rx_sva.sv | BUFFER_COUNT_IN_RANGE, NO_BUFFER_OVERFLOW, NO_RECEIVE_WITHOUT_RUN, STOP_TRANSITION_VALID, ACTIVATE_TRANSITION_VALID, RUN_TRANSITION_VALID, DEACTIVATE_TRANSITION_VALID, DEACTIVATION_BUFFER_DRAINED, PARITY_ERROR_OBSERVED, PARITY_ERROR_ONLY_WHEN_ENABLED | LINK_ACTIVATION, LINK_DEACTIVATION |
| chi_proto_sva.sv | REQ_INPUT_STABLE_UNDER_BACKPRESSURE, RSP_INPUT_STABLE_UNDER_BACKPRESSURE, DAT_INPUT_STABLE_UNDER_BACKPRESSURE, REQ_STABLE_UNDER_BACKPRESSURE, RSP_STABLE_UNDER_BACKPRESSURE, SNP_STABLE_UNDER_BACKPRESSURE, DAT_STABLE_UNDER_BACKPRESSURE, ACTIVE_RESPONSE_ID_CONSISTENT, RETRY_RESPONSE_ID_CONSISTENT, ONE_COMPLETION_PATH, PCRD_GRANT_FIELDS_UNUSED, RETRY_ACK_GRANT_MUTUALLY_EXCLUSIVE | RETRY_ACK_THEN_PCRD_GRANT_CLASS, PCRD_GRANT_SENT, DCT_SNOOP_FORWARD_PATH, DIRECTORY_BACK_INVALIDATION, EXCLUSIVE_SUCCESS, BASIC_READ_FLOW, BASIC_WRITE_FLOW |
| chi_mn_sva.sv | REQ_INPUT_STABLE_UNDER_BACKPRESSURE, RSP_INPUT_STABLE_UNDER_BACKPRESSURE, SNP_STABLE_UNDER_BACKPRESSURE, RSP_STABLE_UNDER_BACKPRESSURE, DVM_RESPONSE_ID_CONSISTENT | DVM_COMPLETION |
| chi_ordering_sva.sv | pairwise NO_DUPLICATE_ACTIVE_LINE, per-entry ALLOCATION_LINE_NOT_ACTIVE | none |
| chi_hnf_ordering_sva.sv | ORDERED_REQUEST_STARTS_QUIESCENT, ACTIVE_ORDERED_REQUEST_SERIALIZES | none |

Table 12.1 Bound SVA files with assertion and cover counts

These are local assertions. They do not check complete opcode legality, credit conservation across the whole port, cache-state correctness, every response dependency, or end-to-end forward progress.

Binding an assertion module that takes an unpacked array port is not portable across simulators, so chi_pos exports its line-address array packed. This is the reason chi_pos.lineAddr, chi_hazard.activeLineAddr and chi_ordering_sva.lineAddr are packed two-dimensional rather than unpacked.

## 12.2 Functional coverage outline

A complete environment should cross topology, channel, source and destination, QoS, backpressure length and route length; cover every supported request, response, snoop and data opcode; and exercise the SAM first-match, security and update cases. It should hit every Home state, snoop-filter hit, miss and replacement, hazard reason, retry class, credit grant and return, exclusive success and failure, DCT eligibility and fallback, DMT and DWT selection, stash target and Data Pull accept and decline, PrefetchTgt mapping, DVM target count, APB error and CHI-MMIO offset. Negative bins should include unknown NodeIDs, illegal configuration, malformed completion identifiers, flit parity faults and credit-contract violations.

## 12.3 UVM structure

chi_env
├── rn_agent[*] / sn_agent[*]      active node-facing chi_if agents
│                                  including L-Credit and activation
├── apb_agent                      active only for CFG_APB
├── protocol_scoreboard            TxnID, DBID, data and completion
├── directory_reference_model      architectural RN states and sharers
├── sam_reference_model            regions, security, hash, commit
├── topology_route_scoreboard      NodeID, channel preservation, ordering
├── link_credit_scoreboard         per-channel credit conservation
├── virtual_sequencer              reads, writes, atomics, snoops, DVM,
│                                  retry, stash
└── coverage_collector

Agents must apply the node-class channel rules from Section 4 rather than drive every signal in the superset, and they must respect L-Credits: a flit may be sent only while the link is in RUN and a credit is held.

## 12.4 Formal-friendly properties

Recommended proof targets are: ready/valid stability; no duplication or loss in each topology; destination and channel preservation; one-hot grants; bounded age service under permanently ready outputs; XY monotonicity; ring slot conservation; L-Credit conservation across each link direction; SAM first-match determinism; commit atomicity; no duplicate Home line ownership; sequential snoop target uniqueness; retry records never double-booked; outstanding P-Credits never exceeding the granted resource; completion identifiers matching the accepted request; and reset returning all valid state to empty. Liveness assumptions must state eventual downstream ready and eventual legal response.

## 12.5 Source manifest

filelist.f is the authoritative compile order:

pkg/chi_cfg_pkg.sv
pkg/chi_pkg.sv
if/chi_if.sv
link/chi_lcrd_mgr.sv
link/chi_link_fsm.sv
link/chi_link_tx.sv
link/chi_link_rx.sv
link/chi_link_port.sv
noc/chi_vc_arb.sv
noc/chi_xbar.sv
noc/chi_ring.sv
noc/chi_route_compute.sv
noc/chi_router.sv
noc/chi_mesh_xp.sv
noc/chi_mesh.sv
noc/chi_noc_top.sv
sam/chi_hn_hash.sv
sam/chi_sam.sv
sam/chi_prefetch_map.sv
hni/chi_cfg_slave.sv
hni/chi_hni_top.sv
mn/chi_mn_dvm.sv
hnf/chi_pos.sv
hnf/chi_hazard.sv
hnf/chi_snoop_filter.sv
hnf/chi_snp_gen.sv
hnf/chi_retry.sv
hnf/chi_dct_ctrl.sv
hnf/chi_atomic_alu.sv
hnf/chi_dmt_ctrl.sv
hnf/chi_dwt_ctrl.sv
hnf/chi_stash_ctrl.sv
hnf/chi_cmo.sv
hnf/chi_excl_mon.sv
hnf/chi_hnf_top.sv
top/chi_interconnect_top.sv
top/chi_interconnect_compile.sv
top/chi_link_compile.sv
top/chi_hnf_compile.sv
sva/chi_link_sva.sv
sva/chi_link_rx_sva.sv
sva/chi_proto_sva.sv
sva/chi_mn_sva.sv
sva/chi_ordering_sva.sv
sva/chi_hnf_ordering_sva.sv
top/chi_sva_compile.sv

top/chi_stage1_compile.sv, top/chi_sam_compile.sv, top/chi_xbar_compile.sv, top/chi_ring_compile.sv and top/chi_mesh_compile.sv are supplemental elaboration harnesses. They are not entries in filelist.f and are not product hierarchy.

tb/chi_node_link_bfm.sv, tb/chi_sim_harness.sv and tb/chi_smoke_tb.sv are the simulation files described in Section 12.7. They are also outside filelist.f, and they are not synthesizable. Makefile drives lint, the smoke run and document generation.

## 12.6 Passing lint commands

chi_interconnect_compile is the lint and elaboration target for the product top. The product top’s node-facing ports are chi_if arrays, and a tool that elaborates chi_interconnect_top as the design root has no interface instances to bind them to, so the harness creates them.

verilator --lint-only --assert --sv -Wall -Wno-fatal \
  -Wno-UNUSEDSIGNAL -Wno-UNUSEDPARAM -Wno-UNDRIVEN -Wno-BLKLOOPINIT \
  -f filelist.f --top-module chi_interconnect_compile

verilator --lint-only --assert --sv -Wall -Wno-fatal \
  -Wno-UNUSEDSIGNAL -Wno-UNUSEDPARAM -Wno-UNDRIVEN -Wno-BLKLOOPINIT \
  -f filelist.f --top-module chi_sva_compile

Both commands produce no warnings, including no UNOPTFLAT, so the design has no circular combinational logic. UNUSEDSIGNAL and UNUSEDPARAM are suppressed because superset protocol records, optional ports and compile harnesses intentionally expose fields that a given configuration does not consume. UNDRIVEN is suppressed because the lint harnesses do not instantiate external node drivers for chi_if. BLKLOOPINIT is suppressed for Verilator 5.020’s restriction on nonblocking initialization loops over the unpacked snoop-filter arrays. Wno-fatal reports remaining warnings without turning them into failures.

A supplemental harness is checked by appending its file and selecting it as the top, for example:

verilator --lint-only --assert --sv -Wall -Wno-fatal \
  -Wno-UNUSEDSIGNAL -Wno-UNUSEDPARAM -Wno-UNDRIVEN -Wno-BLKLOOPINIT \
  -f filelist.f top/chi_ring_compile.sv --top-module chi_ring_compile

The harnesses chi_hnf_compile, chi_link_compile, chi_mesh_compile, chi_ring_compile, chi_xbar_compile, chi_sam_compile and chi_stage1_compile all pass with the same options.

## 12.7 Reset, link bring-up and idle smoke run

Lint elaborates the design but never executes it, so neither the chi_cfg_check elaboration checks of Section 3.3 nor any bound assertion actually runs during a lint pass. tb/chi_smoke_tb.sv closes that gap:

make sim

This is not a protocol testbench. There is no traffic generator, scoreboard or coverage model. It checks three things.

First, the elaboration checks execute. This is what confirms at runtime that the default configuration is legal, that every node has a unique placement, that each NodeID’s routing fields decode back to the placement that produced it, and that all four derived flit widths fall inside the Issue H ranges.

Second, every link comes up. The interconnect owns only its half of each link: it initiates the direction it transmits on and acknowledges the direction it receives on. Without a peer on the node side, every receive link stays in STOP and no L-Credit is ever issued, so chi_node_link_bfm supplies that peer. It is the mirror image of chi_link_port: it initiates the node transmit direction, acknowledges the interconnect’s, issues L-Credits on the channels the node receives, and counts and discards arriving flits. chi_sim_harness attaches one model per external port. The run requires all eight external links to reach RUN in both directions and to hand over their full lcrdDepth allocation on every applicable channel, and requires the HN-F, HN-I and MN ordinals to report no link, since those are internal.

Third, the fabric is silent when idle. With the links up and credits available, the interconnect is free to transmit, so the run fails if any node receives a flit while no request has been made. It also fails on any elaboration $fatal, any bound assertion firing out of reset, any reported flit parity or L-Credit error, and any link dropping out of RUN.

Node and channel counts are derived from CHI_CFG_DEFAULT rather than written as literals, so the testbench follows a reconfigured node list without edits. chi_sim_harness rather than chi_interconnect_compile is the simulation top, because the compile harness deliberately leaves the node-driven interface inputs undriven for lint.

The run is repeated with randomised initialisation:

make simrand

Verilator initialises state to zero, so anything that depends on a register or an input having been driven passes by accident. A four-state simulator propagates X instead and fails. make simrand approximates that by replaying the same run under several initialisation seeds, and it is the only part of this flow that can catch a missing driver. It found one: a node class configured to zero still has a one-element sentinel interface, because an interface array cannot be declared empty, and the port loop originally built a complete link layer for that sentinel. Nothing drove its node-side inputs. The loop now runs to the configured count, so no link is built for a class that does not exist.

## 12.8 Tool dependence of this code

The flow above uses Verilator, which is not equivalent to a commercial simulator. Three consequences are visible in the source and should be understood before this RTL is taken into a Questa, Xcelium or VCS flow.

First, some structure exists to satisfy Verilator rather than the language. UNOPTFLAT is reported per procedural block, not per signal, so a single always_comb that writes both directions of a handshake is flagged even when no dependency actually closes. That is why chi_mesh splits into four blocks and why the Home and HN-I ready outputs sit in blocks of their own. The underlying rule those splits express, in Section 2.8, is sound and worth keeping; the block count is an artifact. chi_link_port mirrors interface members into local nets before using them as instance pins, chi_pos exports a packed rather than unpacked array because binding an unpacked array port fails, and the NodeID helpers hoist loop bounds into locals because a function call in a loop condition is miscompiled. None of these change behaviour, and all remain valid SystemVerilog, but none would have been written this way against a commercial tool.

Second, the bound assertions are limited by what Verilator accepts. They use only implication, $past and $stable. Sequence repetition, throughout, within and local variables in sequences are unavailable, and those are exactly what an end-to-end property needs. A property such as “every accepted request eventually receives a completion carrying the matching TxnID” cannot be expressed here, which is the concrete reason Section 12.1 describes the assertion set as local.

Third, Verilator is two-state. It cannot report an X, so uninitialised state, missing resets and undriven inputs do not fail; make simrand is a partial substitute, not a replacement. Neither is there timing, SDF, glitch or multi-clock behaviour, so Section 11 is unverified by this flow. Re-running the elaboration checks and the smoke test under a four-state simulator with X-initialisation is the first thing to do in a commercial flow, and it should be expected to find defects this flow cannot see.

## 12.9 Regenerating this document

The Word file in docs/ is generated from doc-src/:

make doc

which runs:

pandoc doc-src/chi_interconnect_design_spec.md --toc --toc-depth=2 \
  -o docs/chi_interconnect_design_spec.docx

Section numbers are written into the headings rather than generated, so that the cross-references in the prose stay correct in every output format.

# Chapter 13
	OPEN ISSUES, KNOWN LIMITATIONS AND DECISIONS

## 13.1 Known limitations

- Exactly one interface per node and one channel replica are accepted. There is no top-level interface or channel replication.

- Per-hop CHI L-Credits do not exist inside the fabric. The internal topology uses ready/valid with registered crosspoint outputs, and CHI L-Credits terminate at the node ports.

- Only flit parity is live. The scalar *CHK control and credit checks are declared and driven to zero.

- The Home, the HN-I and the MN globally serialize transactions, so the configured POS depth does not provide Home parallelism.

- No RN cache controller is provided; Section 10 is the contract, not RTL.

- MultiReq and CacheLineID, PBHA, MTE tag semantics and MEC access control are carriage only. MPAM is carried but not enforced.

- Atomics are always executed at the Home rather than negotiated to the Subordinate, the atomic datapath is 64 bits so wider operands are unsupported, and DCT has no fallback path.

- DVM decodes no payload, so no operation-specific ordering is applied.

- Runtime SAM writes lack complete validation, CHI-MMIO lacks security and quiescent commit, and APB quiescence does not inspect every internal topology slot.

- An unknown fabric destination is back-pressured indefinitely and has no error sink.

- System coherency and link power sequencing are placeholders.

- There is no full protocol testbench, no formal proof closure, no interoperability evidence, no CDC signoff, no synthesis signoff and no performance characterization.

## 13.2 Implementation-defined decision log

| Decision | Current choice | Consequence |
| --- | --- | --- |
| Node-facing interfaces | One superset chi_if | Applicability is documented and tied off; direction is not modport-enforced. |
| Zero logical node count | A one-element sentinel array | The integrator ties the inactive sentinel outputs low. |
| Internal flow control | Ready/valid with registered crosspoint outputs | No claim of per-hop CHI L-Credits. |
| Handshake structure | Valid never depends on its ready; forward and backward directions live in separate blocks | The fabric elaborates with no circular combinational logic. |
| P-Credit types | 4 of the 16 Issue H values, indexed by request category | Resources are partitioned by category rather than per requester. |
| Retry record capacity | retryRecords entries; back-pressure when full | Credit accounting stays exact instead of over-promising. |
| PrefetchTgt target | A separate address-to-SN-F mapper | The hint reaches memory without consuming Home resources. |
| HN selection | XOR-folded granule modulo numHnf | Supports non-power-of-two Home counts. |
| SAM overlap | The lowest region index wins | A security rejection at the first match does not fall through. |
| NodeID | {Y, X, port, ordinal} | Placement and route fields must agree. |
| Mesh route | Deterministic XY | Simple dependency ordering, no adaptive balancing. |
| Ring tie | Clockwise | Deterministic equal-distance selection. |
| Snoop and DVM fanout | Sequential unicast | Minimal replication state, additive target latency. |
| Home to subordinate mapping | Index modulo the subordinate count | Static striping rather than address-based balancing. |
| Atomic execution point | Always the Home | No far-atomic negotiation, but one serialization point. |
| APB security | Writes require PPROT[1]==0 | Reads remain unrestricted. |
| Reconfiguration | Staged fields plus an explicit commit | Software owns the ordering; CHI-MMIO is less protected than APB. |
| Reset | Synchronous active low | No asynchronous reset behaviour and no CDC support. |
| Bound assertion array ports | Packed two-dimensional, never unpacked | bind elaborates portably across simulators. |
| Loop bounds in helper functions | Computed once into a local, never a call in the loop condition | The bound is not recomputed per iteration, and the design simulates. |

Table 13.1 Implementation-defined decisions and their consequences